Draft ETSI TS 103 998 V1.1.1 (2023-07)

Publicly Available Specification (PAS);

DASH-IF:

Content Steering for DASH

Technical Specification

Reference

DTS/JTC-117

Keywords

broadband, CDN, DASH, internet

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - APE 7112B

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° w061004871

Important notice

The present document can be downloaded from:
https://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

If you find a security vulnerability in the present document, please report it through our

Coordinated Vulnerability Disclosure Program:

https://www.etsi.org/standards/coordinated-vulnerability-disclosure

Notice of disclaimer & limitation of liability

The information provided in the present deliverable is directed solely to professionals who have the appropriate degree of experience to understand and interpret its content in accordance with generally accepted engineering or

other professional standard and applicable regulations.

No recommendation as to products and services or vendors is made or should be implied.

No representation or warranty is made that this deliverable is technically accurate or sufficient or conforms to any law and/or governmental rule and/or regulation and further, no representation or warranty is made of merchantability or fitness for any particular purpose or against infringement of intellectual property rights.

In no event shall ETSI be held liable for loss of profits or any other incidental or consequential damages.

Any software contained in this deliverable is provided "AS IS" with no warranties, express or implied, including but not limited to, the warranties of merchantability, fitness for a particular purpose and non-infringement of intellectual property rights and ETSI shall not be held liable in any event for any damages whatsoever (including, without limitation, damages for loss of profits, business interruption, loss of information, or any other pecuniary loss) arising out of or related to the use of or inability to use the software.

Copyright Notification

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restrictions extend to reproduction in all media.

© ETSI 2023.

© European Broadcasting Union 2023.

All rights reserved.

Contents

Intellectual Property Rights	4

Foreword	4

Modal verbs terminology	4

Executive summary	4

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Overview and Architecture	6

5	Content Steering Signalling in DASH	8

5.1	Introduction	8

5.2	Signalling Requirements and Recommendations	9

6	DASH Steering Manifest and Server Behaviour	10

6.1	Overview	10

6.2	JSON Syntax	10

6.3	Semantics	10

7	Normative DASH Client Steering behaviour	11

8	Extended HTTP GET request parametrization instructions	14

8.1	URL Query information for Content Steering	14

8.2	Updates to Annex I - Flexible Insertion of URL Parameters	15

8.2.1	Introduction	15

8.2.2	Modified content steering server URLs building process	15

Annex A (informative):	Example implementations	16

A.1	Basic workflow example	16

A.2	Advanced steering example	17

A.3	Pathway cloning example	18

Annex (informative):	Change History	21

History	22

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The declarations pertaining to these essential IPRs, if any, are publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI Directives including the ETSI IPR Policy, no investigation regarding the essentiality of IPRs, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members. 3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners. oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and of the oneM2M Partners. GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

# Foreword

This Technical Specification (TS) has been produced by Joint Technical Committee (JTC) Broadcast of the European Broadcasting Union (EBU), Comité Européen de Normalisation ELECtrotechnique (CENELEC) and the European Telecommunications Standards Institute (ETSI).

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Executive summary

Content distributors often use multiple Content Delivery Networks (CDNs) to distribute their content to the end-users. They may upload a copy of their catalogue to each CDN, or more commonly have all CDNs pull the content from a common origin. Alternate URLs are generated, one for each CDN, that point at identical content. DASH players may access alternate URLs in the event of delivery problems. Content steering describes a deterministic capability for a content distributor to switch the content source that a player uses either at start-up or midstream, by means of a remote steering service.

# 1	Scope

The present document specifies Content Steering for DASH.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, DASH-IF cannot guarantee their long-term validity.

The following referenced documents are necessary for the application of the present document:

[1]	HTTP Live Streaming 2nd Edition.

[2]	ISO/IEC 23009-1:2022/CDAM2:2023: "Information technology -- Dynamic adaptive streaming over HTTP (DASH) - Part 1: Media presentation description and segment formats" (available as MDS22328-WG03-N00804).

[3]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[4]	IETF RFC 9110: "HTTP Semantics".

[5]	IETF RFC 6585: "Additional HTTP Status Codes".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document, but they assist the user with regard to a particular subject area.

[i.1]	DASH-IF Interoperability Points, Part 1: Overview, architecture and interfaces.

[i.2]	ISO/IEC 14496-12:2021: "Information technology -- Coding of audio-visual objects -- Part 12: ISO base media file format".

[i.3]	ISO/IEC 23000-19:2020: "Information technology -- Multimedia application format (MPEG-A) -- Part 19: Common media application format (CMAF) for segmented media".

[i.4]	CTA-5004: "Web Application Video Ecosystem-Common Media Client Data".

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the following terms apply:

Content Steering Server: network element that provides steering information to one or several or many DASH Players for DASH operation across multiple CDNs

DASH Content Steering Manifest: document that includes steering instructions to a DASH player provided by a Content Steering Server

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CDN	Content Delivery Network

CMCD	Common Media Client Data

CTA	Consumer Technology Association

DASH	Dynamic Adaptive Streaming over HTTP

DCSM	DASH Content Steering Manifest

HTML	HyperText Markup Language

HTTP	HyperText Transfer Protocol

ISO	International Standards Organization

MPD	Media Presentation Description

TTL	Time-To-Live

URL	Uniform Resource Locator

# 4	Overview and Architecture

Content distributors often use multiple Content Delivery Networks (CDNs) to distribute their content to the end-users as shown in Figure 4-1. They may upload a copy of their catalogue to each CDN, or more commonly have all CDNs pull the content from a common origin. In the DASH Media Presentation Description (MPD) as defined in ISO/IEC 23009-1 [2], multiple URLs are provided, one for each CDN, that point at identical content. Typically, a DASH player will access content from one single location, using the default location defined by the MPD.

If the DASH player then observes delivery problems, it may choose to access content via the alternate URLs. This operation is completely client-driven, is not standardized between players and may not be the desired behaviour of the content distributor.

Content steering provides a deterministic capability for a content distributor to switch the content source that a player uses either at start-up or midstream, by means of a remote steering service.

Figure 4-1: Basic Architecture with Content Steering

Steering is accomplished by having the DASH client periodically access a content steering server to retrieve a steering manifest, which instructs the player as to the availability and priority of content sources.

The typical procedures followed when content steering is in use are shown in Figure 4-2 for the case when the content is provided on two CDNs. The DASH content provider generates an MPD that includes Base URLs to CDN1 and CDN2, as well as an address where the clients can access the content steering server. The provider also uploads the MPD and the Content segments to both CDNs. At the start of playback, the DASH client requests the MPD from one CDN, in this case from CDN2. It finds the content steering server URL, and it may find information that instructs it to contact the content steering server prior to the first segment request versus the default behaviour of making the request once its starting buffer is full. The player then makes a request to the content steering server. The content server responds with a content steering manifest and the DASH client uses the information within to prioritize the segment source, in this case from CDN2. After some time, the content provider may collect operational information from the participating clients, for example by using Common Media Client Data (CMCD) as defined in CTA-5004 [i.4]. Based on this information, the content provider may update the content steering server, and based on this updated information, the content steering manifest may change. When the client requests an update to the content steering manifest, new information may be provided that instructs the DASH client to request the Segments from CDN1 instead of CDN2. The DASH client then switches smoothly, at a segment boundary, to download the Segments from CDN1 instead of CDN2. The steering server response can also be used to steer the DASH client between alternate sources for DASH manifest refreshes, via service descriptors contained within the MPD.

Figure 4-2: Typical procedures in Content Steering operation

The present document provides the following detailed information to support interoperable operation of content steering in DASH Media Presentations:

-	Clause 5 provides the details on how to signal different alternate CDNs in the DASH MPD using existing BaseURL elements as well as an extension to the DASH MPD in order to provide the location of the Content Steering server as well as some additional instructions to the client based on ISO/IEC 23009-1:2022/Amd.2 [2] using the ContentSteering element as well as the Service Location functionality.

-	Clause 6 provides the details on the DASH Content Steering Manifest (DCSM) and the behaviour of the content steering server, in particular the semantics and JSON syntax of the steering server response.

-	Clause 7 defines the DASH player behaviour in detail. A DASH player supporting Content Steering is required to implement the detailed procedures documented in this clause.

-	Clause 8 provides guidelines on the usage of DASH Annex I of ISO/IEC 23009-1 [2] for providing the client identity as part of the DASH player to content steering server communication.

-	Annex A provides operational examples of a DASH MPD as well as of the response of the content steering server.

# 5	Content Steering Signalling in DASH

## 5.1	Introduction

Content distributors often use multiple Content Delivery Networks (CDNs) to distribute their content to the end-users. They may upload a copy of their catalogue to each CDN, or more commonly have all CDNs pull the content from a common origin. Alternate URLs are generated, one for each CDN, that point at identical content.

The DASH MPD supports the BaseURL element to allow the listing of these alternate URLs, as well as the Location and PatchLocation element for pointing at alternate manifest sources. DASH players may access alternate URLs in the event of delivery problems. Content steering describes a deterministic capability for a content distributor to switch the content source that a player uses either at start-up or midstream, by means of a remote steering service.

To enable Content Steering the following functionalities are provided in MPEG DASH.

Content Steering information can be provided to the DASH client including the following information according to Annex K.3 of ISO/IEC 23009-1:2022/Amd.2 [2]:

ContentSteeringServer:	A URL that can be used to access the Content Steering server. The URL references to a DASH Content Steering Manifest as defined in clause 6 of the present document.

defaultServiceLocation: This parameter specifies a space-separated list of Service Locations as defined in clause 5.6.6 of ISO/IEC 23009-1:2022/Amd.2 [2] that the client should use to access the selected resources, in case multiple access exist. This for example applies, when no content steering server is available, or before a valid response from a content steering server is available.

queryBeforeStart: This flag, if set to true, indicates that the player is expected to resolve the response from the Steering Server prior to starting playback. The default value is false.

clientRequirement:	The flag, if true, indicates that the client, if it is in the context of the Service Description shall follow the content steering rules as defined in clause 7 of the present document. If false, indicates to client that it is its own decision to make use of content steering operation or not.

This content steering information can be provided as part of the MPD, either in the MPD directly or as part of a Service Description. The latter allows to target only a subset of specific clients that follow the service description. The MPEG-DASH specification does not prevent that the above content steering information is provided independent of the MPD, for example directly from the application using a configuration API for the DASH access engine as shown in clause K.2 of ISO/IEC 23009-1:2022 [2].

Service Location as introduced in clause 5.6.6 of ISO/IEC 23009-1:2022/Amd.2 [2] allows to bind two or more resources a common network location, for example a common Content Delivery Network. This information may be attached to a BaseURL element, a Location element or a PatchLocation element.

Flexible URL of URL Parameters as defined in Annex I of ISO/IEC 23009-1:2022/Amd.2 [2], for which the request to a Content Steering server may add specific query parameters.

## 5.2	Signalling Requirements and Recommendations

Based on the functionalities in ISO/IEC 23009-1:2022/Amd.2 [2], the following requirements hold for signaling Content Steering in a DASH MPD in the context of the present document.

A ContentSteering element shall be present either on MPD level if all clients are targeted, or, if a specific set of clients are targeted, as part of a ServiceDescription element. The ContentSteering element shall appear at most once in a parent element. The value of the element shall reference to a DASH Content Steering Manifest (DCSM) as defined in clause 6.

If the resolved resource is not a Content Steering Manifest (DCSM), the resource is ignored.

The optional attributes @defaultServiceLocation, @queryBeforeStart and @clientRequirement may be used according to their semantics.

The @serviceLocation attribute may be present in BaseURL, Location and PatchLocation elements. If present and used in a content steering manifest, the string value of the @serviceLocation shall only contain characters from the set [a..z], [A..Z], [0..9], '.', '-', and '_'.

NOTE:	Overlapping @serviceLocation names for steerable and non-steerable BaseURLs is discouraged. i.e. if a server-side ad insertion provides BaseURL elements with an overlapping @serviceLocation value it could cause unintended behavior.

# 6	DASH Steering Manifest and Server Behaviour

## 6.1	Overview

Based on the description in clause 4, the DASH Steering server provides a Steering Manifest on a request from a DASH client. The DASH Content Steering Manifest (DCSM) is a json document and shall be formatted according to the JSON schema provided in clause 6.2.

The semantics of the key-value pairs of the DCSM are defined in clause 6.3.

A client shall ignore any key of the DCSM that it does not recognize. DCSM keys are case-sensitive.

NOTE:	This structure is intentionally similar to that defined by [1] Section 7.1 for HLS for the purposes of interoperability. The DASH variant and versioning are defined in the present document and intentionally exclude features in the HLS design which are not applicable to DASH.

## 6.2	JSON Syntax

{ "VERSION": number, // REQUIRED, must be an integer "TTL": number, // REQUIRED, number of seconds "RELOAD-URI": string, // OPTIONAL, URI "PATHWAY-PRIORITY": [] // REQUIRED, array of serviceLocation identifiers in order of preference "PATHWAY-CLONES": // OPTIONAL, array of one or more Pathway Clone objects [ { "BASE-ID": string, // REQUIRED. Pathway ID of the Base Pathway "ID": string, // REQUIRED. Pathway ID for the Pathway Clone "URI-REPLACEMENT": // REQUIRED. URI replacement rules { "HOST": string, // OPTIONAL. Hostname for cloned URIs "PARAMS": // OPTIONAL. Query parameters for cloned URIs { JSON object where keys are query parameter names and values are query parameter values. Any reserved characters in the strings shall be percent encoded [RFC3986] } } } ] }

## 6.3	Semantics

The semantics of the DCSM are defined in Table 6.3-1.

Table 6.3-1: Semantics of the DASH Content Steering Manifest

| Key |  |  | Use | Description |
| --- | --- | --- | --- | --- |
|  | DCSM |  |  | A URL that can be used to access the Content Steering server. The URL points to a DASH Content Steering Manifest (DCSM) as defined in clause 6. |
|  |  | VERSION | M | The version of DCSM. This specification defines DASH Steering Manifest version 1. A client shall refuse to use a DCSM with an unrecognized value for this key. |
|  |  | TTL | M | Specifies how many seconds the client shall wait before reloading the DCSM. The recommended value is 300 seconds. The Steering Server may vary the TTL by client and the TTL may vary with each reload of the steering server manifest. |
|  |  | RELOAD-URI | O | If present, specifies the URI the client shall use the next time it obtains the DCSM. The RELOAD-URI may be relative to the current DCSM Manifest URI. If not present, the current DCSM Manifest URI shall be used. |
|  |  | PATHWAY-PRIORITY | OD Default [] | PATHWAY-PRIORITY is an array of BaseURL@serviceLocation and Location@serviceLocation IDs. Elements in the PATHWAY-PRIORITY array are ordered by preference of the serviceLocation to be selected, with the first being most preferred. If present, the value of this key shall contain at least one element for serviceLocation. A @serviceLocation ID in the PATHWAY-PRIORITY array shall not appear more than once. Clients shall ignore unrecognized @serviceLocation IDs in the PATHWAY-PRIORITY array. |
|  |  | PATHWAY-CLONES | OD Default [] | PATHWAY_CLONES is an array of pathway clones. A pathway clone defines a virtual BaseURL@serviceLocation or Location@serviceLocation. A Pathway Clone is produced by taking an existing serviceLocation and applying well-defined replacements to the URIs of every serviceLocation member. If present, the value of this key shall contain at least one element defining a pathway clone. Any reserved characters in the query parameters for cloned URIs shall be percent encoded according to RFC3986 [3]. |
| Key M=mandatory, O=optional, OD=optional with default value, CM=conditionally mandatory |  |  |  |  |

# 7	Normative DASH Client Steering behaviour

A DASH client supporting DASH Content Steering as defined in the present document shall adhere to the following procedures:

- If the ContentSteering element is present in the MPD and the @clientRequirement is set to FALSE, the DASH client shall either ignore the ContentSteering element or it shall follow the requirements as if the @clientRequirement is set to TRUE.

- If the ContentSteering element is present in the MPD and the @clientRequirement is not set to FALSE, then the client shall parse the element , extract the server URI, as well as the @defaultServiceLocation,  and @queryBeforeStart attributes, if present, and shall process as follows in the remainder of this clause..

- If any extended HTTP GET request parametrization instructions as defined in clause 8 are present in the MPD which target the ContentSteering element, then they shall be executed at this stage to modify the server URI.

- The client shall use the server URI as the STEERING-SERVER-URL.

- If @queryBeforeStart is absent, or present and set to false, then the client shall follow its default start-up sequence. Once playback has started and the client has reached its target buffer, it shall proceed to the next step.

- The client shall make a GET request to the STEERING-SERVER-URL.

- The GET request should be accompanied by the two optional query parameters shown below.

The _DASH_pathway parameter shall contain a value of the currently selected BaseURL@serviceLocation, contained in double-quotes. If playback has not yet started due to this being the first request with @queryBeforeStart set to true, then the_DASH_pathway parameter shall be omitted. If multiple @serviceLocation attributes are being currently matched, for example in the Location element or in a BaseURL element, or if multiple @serviceLocation attributes have been matched since the last steering server request, then all matched @serviceLocation attributes shall be listed, separated by a comma delimiter and contained within a single set of double-quotes.

The _DASH_throughput parameter represents a current prediction of media download throughput observed by the client, in units of integer bits per second, from the applied @serviceLocation. The exact method of bit rate estimation may vary by client. If the client has multiple throughput estimates available, for example from demuxed audio and video downloads, then it should report the higher of the available estimates. If playback has not yet started due to this being the first request with @queryBeforeStart set to true, or if the throughput value is unknown for other reasons, then the_DASH_throughput parameter should be omitted. If the _DASH_pathway parameter value references multiple serviceLocations, then the _DASH_throughput parameter value shall consistent of a comma separated list of throughput values, sequenced such that the n'th item in the _DASH_throughput list applies to the n'th item in the _DASH_pathway list.

NOTE:	The client may send additional useful information via query arguments to the steering server, such as the transfer latency between client and server. It should not use the _DASH_ prefix to transfer this information and such communication would constitute a private relationship between the distributor and the steering server service. Distribution vendors will collect constant real-time QoE information from their clients which they will use to make steering server decisions and it is not anticipated that the core steering decisions will be based solely on information provided by clients as they make steering server requests.

Upon receipt of the steering server response, the client should parse it and retrieve the VERSION, TTL, PATHWAY-PRIORITY array, the optional RELOAD-URI and the optional PATHWAY-CLONES array. The client shall ignore any steering manifest keys it does not recognize. Manifest keys are case-sensitive.

The client sets a timer to re-request the STEERING-SERVER-URL after TTL seconds.

If RELOAD-URI is present, then the client shall update the STEERING-SERVER-URL to match that specified by RELOAD-URI. The RELOAD-URI may be relative to the current server URI.

If the VERSION is a value other than 1, then the client shall abort any further steering behavior.

The string entries in the PATHWAY-PRIORITY array represent a prioritized list of serviceLocations from which playback should take place, with the highest priority option listed first. This highest priority item is termed the preferred service location. The PATHWAY-PRIORITY array is applicable across all periods present in the manifest as long as they include one or more BaseURL elements with a @serviceLocation attribute value included in the PATHWAY-PRIORITY array, or inherit from top level BaseURL elements with a @serviceLocation attribute value included in the PATHWAY-PRIORITY array. The PATHWAY-PRIORITY array may also reference a serviceLocation which is defined by the ID attribute of a PATHWAY-CLONE.

If the preferred service location is one defined by a PATHWAY-CLONE, then the client must synthesize the new serviceLocation by cloning an existing serviceLocation. The serviceLocation it shall clone is defined by the PATHWAY-CLONES@BASE-ID attribute. The client would process the BaseURL or Location element URI as normal, but then make two changes to the generated URI. Firstly, it would substitute the HOST component of the URI with the new host, defined by the URI-REPLACEMENT@HOST attribute. Secondly, if the optional URI-REPLACEMENT@PARAMS attribute is present, then for each item in the PARAMS object, it would add query arguments to the URL matching the key and value defined by the PARAMS item. These query arguments are appended to any existing arguments which may already be present in the URI due to a template definition, inheritance or Annex I "Extended HTTP GET request parametrization" of ISO/IEC 23009-1 [2]. If a parameter of the same name is already present in the URI, then it shall be replaced by the one from the PARAMS object. A DASH client must ignore all other attributes in the PATHWAY-CLONE object, including any URI-REPLACEMENT.PER-VARIANT-URIS and URI-REPLACEMENT.PER-RENDITION-URIS objects. A pathway clone may use another pathway clone as its base if it appears earlier in the PATHWAY-CLONES array. A client that does not have a serviceLocation specified by the BASE-ID string of a Pathway Clone object shall ignore the Pathway Clone.

If the client is playing content defined by a BaseURL element with a @serviceLocation not equal to the preferred service location, then the client shall switch at the next segment load to retrieving future content from the BaseURL referenced by the preferred service location. Note that existing requests against a prior @serviceLocation should be allowed to complete and forward buffers should not be trimmed. If the preferred service location is not described in the DASH MPD, then the client should attempt to switch to the next highest priority service location. If no @serviceLocation attributes match the manifest, then the client may ignore the current PATHWAY-PRIORITY array and make a default decision about which BaseURL to use. It should still reload the RELOAD-URI after the specified TTL interval in case new service locations are added.

If the manifest contains one or more Location elements with @serviceLocation attributes, then prior to a manifest update, the client shall evaluate the @serviceLocation attributes against the PATHWAY-PRIORITY array and select the Location element with the highest matching @serviceLocation. If no @serviceLocation attributes match then the client may make a default decision about which Location element to select.

A HTTP 410 [4] response from the steering server indicates that access to the steering server is no longer available and that this condition is likely to be permanent. As a result, if the client has a previous valid PATHWAY-PRIORITY array from the steering server, it should continue to enforce that prioritization for the remainder of playback and should cancel any all future reloads . If the 410 response is received on the first steering server request, then the client should abandon all steering behaviors, cancel all future requests and proceed with playback as if the ContentSteering element were not present.

A HTTP 429 response [5] from the steering server indicates that the server has received too many requests. The client should react by substituting the parsed TTL value with the 429 Retry-After value, if present.

If the client encounters playback problems which would normally cause it to try an alternate BaseURL, it may continue to make that local switching decision, while following these constraints:

- The client may only try BaseURL@serviceLocation attributes which were present in the last steering server response.

The client shall try these serviceLocations in the order in which they were prioritized in the last steering server response.

As it switches away from the highest priority @serviceLocation for local performance reasons, it shall exclude that @serviceLocation for a time-limited period equal to the last steering server TTL that it received. Effectively this means that if the next steering server response again assigns the excluded @serviceLocation as the highest priority, the client shall ignore that instruction and instead process the PATHWAY-PRIORITY array as if the excluded @serviceLocation were not present.

If the client encounters playback problems which would normally cause it to try an alternate Location, it may continue to make that local switching decision, while following these constraints:

- The client may only try Location@serviceLocation attributes which were present in the last steering server response. If no @serviceLocation attributes match, then the client may make a default decision as to which Location element to use.

The client shall try these Location@serviceLocation attributes in the order in which they were prioritized in the last steering server response.

As it switches away from the highest priority Location@serviceLocation for local performance reasons, it shall exclude that @serviceLocation for a time-limited period equal to the last steering server TTL that it received. Effectively this means that if the next steering server response again assigns the excluded @serviceLocation as the highest priority, the client shall ignore that instruction and instead process the PATHWAY-PRIORITY array as if the excluded @serviceLocation were not present.

NOTE: The above description does not address the possible presence of @serviceLocation attribute for the PatchLocation element. If present, then the same procedure as for Location@serviceLocation applies. In ISO/IEC 23009-1, the generalized terminology service location is used, which applies to any of the elements, BaseURL, Location and PatchLocation.

# 8	Extended HTTP GET request parametrization instructions

## 8.1	URL Query information for Content Steering

Query arguments attached to the request if the DCSM can be used to initialize content steering parameters.

In order to do so, it is recommended that the URL substitution mechanism and syntax as defined in MPEG-DASH ISO/IEC 23009-1 [2], clause I.3, “Extended HTTP GET request parametrization” is used. In order to fully support functionality, the substitution mechanism needs to be extended. Details of this is provided in clause 8.2.

The following provides guidelines on how to make use of this functionality.

Consider a MPD URL of the following form:

https://cdn.distributor.com/content/common-cachable-manifest.mpd?steeringToken=12345

with relevant contents shown as:

<MPD xmlns:xsi=”http://www.w3.org/2001/XMLSchema-instance” xmlns=”urn:mpeg:dash:schema:mpd:2011” xsi:schemaLocation=”urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd” type=”dynamic” minimumUpdatePeriod=”PT30S” timeShiftBufferDepth=”PT30M” availabilityStartTime=”2022-02-25T12:30:00” minBufferTime=”PT4S” profiles=”urn:mpeg:dash:profile:isoff-live:2011”> <EssentialProperty schemeIdUri=”urn:mpeg:dash:urlparam:2014” xmlns:up=”urn:mpeg:dash:schema:urlparam:2014”> <up:ExtUrlQueryInfo queryTemplate=”token=$query:steeringToken$” useMPDUrlQuery=”true” includeInRequests=”steering”/> </up:ExtUrlQueryInfo> <BaseURL serviceLocation=”alpha”>https://cdn1.example.com/</BaseURL> <BaseURL serviceLocation=”beta”>https://cdn2.example.com/</BaseURL> <Period id=”1”> ... </Period> <ContentSteering defaultServiceLocation=”alpha” queryBeforeStart=”false”>https://steeringservice.com/app/instance1234</ContentSteering> </MPD>

In this example the request to the steering server would be processed as:

https://steeringservice.com/app/instance1234?steeringToken=12345&_DASH_pathway=alpha&_DASH_throughput=5140000

Note that also a @includeInRequests=”*” may be added to URLs to support any kind of URL being extended with query parameters.

## 8.2	Updates to Annex I – Flexible Insertion of URL Parameters

### 8.2.1	Introduction

In order to fully support the functionality of content steering, extensions to ISO/IEC 23009-1:2022/Amd.2 [2] are available in clause I.3, “Extended HTTP GET request parametrization”. These include

-	Updates to Table I.3 of ISO/IEC 23009-1:2022/Amd.2 [2] in order to add steering ("steering") and any ("*") requests.

-	Updates to clause I.2.4.5 of ISO/IEC 23009-1:2022/Amd.2 [2] adding an example.

### 8.2.2	Modified content steering server URLs building process

In order to create modified content steering server URLs for example as:

https://steeringservice.com/app/instance1234?token=1234&sessionID=h48djn&_DASH_pathway=beta&_DASH_throughput=5140000

The _DASH_ parameters are added automatically by the player as part of the processing rules for Content Steering according to clause 7 and are independent of the URL building mechanisms described in Annex I of ISO/IEC 23009-1 [2].

Annex A (informative):
Example implementations

# A.1	Basic workflow example

This case illustrates service location changes along with performance override.

A DASH MPD is presented to a player.

<MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="dynamic" minimumUpdatePeriod="PT30S" timeShiftBufferDepth="PT30M" availabilityStartTime="2022-02-25T12:30:00" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011"> <BaseURL serviceLocation="alpha">https://cdn1.com/</BaseURL> <BaseURL serviceLocation="beta">https://cdn2.com/</BaseURL> <Period id="1"> <AdaptationSet mimeType="video/mp4" codecs="avc1.4D401F" frameRate="30000/1001" segmentAlignment="true" startWithSAP="1"> <BaseURL>video/</BaseURL> … </AdaptationSet> </Period> <ContentSteering defaultServiceLocation="beta" queryBeforeStart="true">https://steeringservice.com/app/instance1234?token=234523452</ContentSteering> </MPD>

The player would parse the ContentSteering element upon receiving the MPD. Since the @queryBeforeStart attribute is present and set to true, instead of starting playback using the @defaultServiceLocation of "beta", it would make a request to the steering server at https://steeringservice.com/app/instance12345. This request would be:

https://steeringservice.com/app/instance1234?token=234523452

Note the _DASH_ params are not attached to this request since the player has not yet started playback. The server may then return the JSON response below:

{ "VERSION": 1, "TTL": 300, "RELOAD-URI": "https://steeringservice.com/app/instance12345?session=abc" "PATHWAY-PRIORITY": ["alpha","beta"] }

The player would recognize that the highest priority serviceLocation specified is "alpha", so it would use the BaseURL construct of https://cdn1.com/ as it begins to request content. The player would then set a timer so that in 300 s, when the throughput it was estimating is 5.14 Mbps, it would again query the steering server, with the URL:

https://steeringservice.com/app/instance12345?session=abc&_DASH_pathway=alpha&_DASH_throughput=5140000

At that time the steering server may return:

{ "VERSION": 1, "TTL": 250, "RELOAD-URI": "https://steeringservice.com/app/instance12345?session=abc" "PATHWAY-PRIORITY": ["beta","alpha"] }

The player would then switch to loading the next media objects using the BaseURL of https://cdn2.com/. 250 s later it would again request the steering service and the cycle would continue until end-of-stream was reached.

Assume that the player would run in to a delivery problem 100 s after the last steering server response. This problem may be triggered by 404 responses, or throughput degradation. The player decides that "beta" is not a good source and makes a local decision to switch to the next highest priority @serviceLocation "alpha" and to blacklist "beta". This blacklist last for a time-period equal to the last TTL received, which is 250 s. 150 s after taking this action, the player calls the steering server and reports the @serviceLocation it is currently playing using the _DASH_pathway parameter:

https://steeringservice.com/app/instance12345?session=abc&_DASH_pathway=alpha&_DASH_throughput=4880000

Since the steering server is stateful, it knows that it last assigned "beta" but the player is now reporting "alpha" implying a client-initiated change. The steering server can take this signal in to account when making its steering decisions. It may however still reply with:

{ "VERSION": 1, "TTL": 250, "RELOAD-URI": "https://steeringservice.com/app/instance12345?session=abc" "PATHWAY-PRIORITY": ["beta","alpha"] }

Since the client has excluded "beta" for performance reasons for 250 s since the switch was made, it processes the PATHWAY-PRIORITY array as if "beta" were not present and continues to play "alpha". At the next steering server response, the exclusion would have expired and the client should apply the conventional processing rules to the response.

# A.2	Advanced steering example

This case illustrates the independent steering of manifests, media segments and advertising content.

A DASH MPD is presented to a player.

<MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="dynamic" minimumUpdatePeriod="PT30S" timeShiftBufferDepth="PT30M" availabilityStartTime="2022-02-25T12:30:00" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011"> <Location serviceLocation="1234">https://manifest-cdn1.com/</Location> <Location serviceLocation="5678">https://manifest-cdn2.com/</Location> <BaseURL serviceLocation="alpha">https://segments-cdn-A.com/</BaseURL> <BaseURL serviceLocation="beta">https://segments-cdn-B.com/</BaseURL> <Period id="Primary-Content-1"> … </Period> <Period id="Ad-break-1"> <BaseURL serviceLocation="ad1">https://ad-server-1.com/</BaseURL> <BaseURL serviceLocation="ad2">https://ad-server-2.com/</BaseURL> … </Period> <Period id="Primary-Content-2"> <BaseURL serviceLocation="gamma">https://segments-cdn-C.com/</BaseURL> <BaseURL serviceLocation="delta">https://segments-cdn-D.com/</BaseURL> … </Period> <Period id="Ad-break-2"> <BaseURL serviceLocation="ad3">https://ad-server-3.com/</BaseURL> <BaseURL serviceLocation="ad4">https://ad-server-4.com/</BaseURL> … </Period> <Period id="Primary-Content-3"> … </Period> <ContentSteering defaultServiceLocation="1234,alpha,ad1"> https://steeringservice.com/app?token=567 </ContentSteering> </MPD>

Since ContentSteering@queryBeforeStart is not TRUE, the player would start by matching the content of the ContentSteering@defaultServiceLocation elements against the available Location and BaseURL options. As a result, it would start with:

- Load media segments within period "primary-content-1" from https://segments-cdn-A.com

- Refresh the manifest from https://manifest-cdn1.com/</

As soon as playback has started and the player's target buffer has been reached, it would make the following request to the content steering server:

https://steeringservice.com/app?token=567&_DASH_pathway="1234,alpha"&_DASH_throughput=32000000,19000000

Assume the steering server then returns the following JSON response for all future refreshes:

{ "VERSION": 1, "TTL": 300, "RELOAD-URI": "https://steeringservice.com/app/instance1234" "PATHWAY-PRIORITY": ["beta","alpha","ad1","delta","5678","gamma","1234","ad4","ad3"] }

Following the rules of selecting the highest matching Location and BaseURL serviceLocation from the PATHWAY-PRIORITY array, along with BaseURL inheritance, the client would:

- Switch to loading media segments within period "primary-content-1" from https://segments-cdn-B.com

- Switch to loading the manifest from https://manifest-cdn2.com/</

- Load media segments within period "ad-break-1" from https://ad-server-1.com/</

- Load media segments within period "primary-content-2" from https://segments-cdn-D.com

- Load media segments within period "ad-break-2" from https://ad-server-4.com

- Load media segments within period "primary-content-3" from https://segments-cdn-B.com

Assuming that the second steering server request (after 300 s) occurs during period "Primary-Content-2", then client would construct the request in the following manner:

https://steeringservice.com/app/instance1234?_DASH_pathway="5678,beta,ad1,delta"&_DASH_throughput=450000,56000000,21000000,32000000

# A.3	Pathway cloning example

This case illustrates the application of Pathway Cloning to synthesize a new serviceLocation. Interaction with Annex I – "Flexible Insertion of URL Parameters" of ISO/IEC 23009-1 [2] is also shown to provide clarity on how the query arguments should be appended.

A DASH MPD is presented to a player at the URL:

http://www.example.com/dash/cloning.mpd?token=1234

with the following content:

<MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="dynamic" minimumUpdatePeriod="PT30S" timeShiftBufferDepth="PT30M" availabilityStartTime="2022-02-25T12:30:00" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011"> <BaseURL serviceLocation="alpha">https://cdn1.com/</BaseURL> <BaseURL serviceLocation="beta">https://cdn2.com/</BaseURL> <EssentialProperty schemeIdUri="urn:mpeg:dash:urlparam:2014" xmlns:up="urn:mpeg:dash:schema:urlparam:2014"> <up:UrlQueryInfo includeInRequests="mpd segment steering" queryTemplate="$querypart$" useMPDUrlQuery="true"/> </EssentialProperty> <Period id="1"> <AdaptationSet mimeType="video/mp4" contentType="video" subsegmentAlignment="true" subsegmentStartsWithSAP="1" par="16:9"> <SegmentTemplate duration="120" timescale="30" media="$RepresentationID$/$RepresentationID$_$Number$.m4v?geo=US" startNumber="1" initialization="$RepresentationID$/$RepresentationID$_0.m4v?geo=US"/> <Representation id="1024x576_2500k" codecs="avc1.64001f" bandwidth="3134488" width="1024" height="576" frameRate="30" sar="1:1" scanType="progressive"/> … </AdaptationSet> </Period> <ContentSteering defaultServiceLocation="beta" queryBeforeStart="true">https://steeringservice.com/app/instance1234?sessionID=64829</ContentSteering> </MPD>

The player would parse the ContentSteering element upon receiving the MPD. Since the @queryBeforeStart attribute is present and set to true, instead of starting playback using the @defaultServiceLocation of "beta", it would make a request to the steering server. This request, after applying the rules of Annex I of ISO/IEC 23009-1 [2], would be:

https://steeringservice.com/app/instance1234?sessionID=64829&token=1234

Note the _DASH_ params are not attached to this request since the player has not yet started playback. The server may then return the JSON response below:

{ "VERSION": 1, "TTL": 300, "RELOAD-URI": "https://steeringservice.com/app/instance12345?session=abc" "PATHWAY-PRIORITY": ["charlie","alpha","beta"] "PATHWAY-CLONES": [ { "BASE-ID": "alpha", "ID": "charlie", "URI-REPLACEMENT": { "HOST": "segments-cdn-charlie.com", "PARAMS": { "token-for-charlie": "dkfs1239414" } } } ] }

The player would recognize that the highest priority serviceLocation specified is "charlie", which is a synthesized serviceLocation built via cloning. The player would parse the PATHWAY-CLONES array to locate the definition of "charlie". It would construct "Charlie" by building the URI for "alpha" as it would normally, but then substituting the HOST component of the URL with "segments-cdn-charlie.com" and also appending the query argument "token-for-charlie". The first request for a media segment would be:

https://segments-cdn-charlie.com/1024x576_2500k/1024x576_2500k_1.m4v?geo=US&token=1234&token-for-charlie=dkfs1239414

The player would then set a timer so that in 300 s, when the throughput it was estimating is 5.14 Mbps, it would again query the steering server, with the URL:

https://steeringservice.com/app/instance12345?session=abc&token=1234&_DASH_pathway=charlie&_DASH_throughput=5140000

The cycle would then continue with the player parsing the steering server response and making the appropriate pathway selection.

Annex (informative):
Change History

| Date | Version | Information about changes |
| --- | --- | --- |
| 2022-07-10 | 0.9.0 | Version published for community review. |
| 2022-12-07 | 0.9.5 | Updates following community review. |
| 2022-12-21 | 0.9.6 | Version submitted for second community review |
| 2023-03-31 | 0.9.7 | Version addressing the comments from the second community review |
| 2023-05-08 | 0.9.8 | Version prepared for IPR Review and ETSI submission |

# History

| Document history |  |  |
| --- | --- | --- |
| V1.1.1 | July 2023 | Pre-processing done before TB approval E-mail: mailto:edithelp@etsi.org |